Arduino code spec

### Component list

- x2 buttons (Calibrate, pause)

- x1 buzzer

- x1 red led light strip

- x1 4 digit timer LED display screen

- X1 scales (x4 load cells in wheatstone bridge configuration)

- x1USB C slot to charge

- X1 Power bank slot

| Function | ESP32-S3 FREENOVE GPI0 Slot |
| --- | --- |
| Calibrate button | 7 |
| Pause button | 6 |
| Buzzer | 8 |
| Red light strip (Via 12V Boost) Green | 21 5 |
| CLK - TM1637 4 digit display | 9 |
| DIO - TM1637 4 digit display | 10 |
| SCK - HX711 Load Cells | 19 |
| DT - HX711 Load Cells | 18 |

### Instructions

- Power quiz master on with power switch underside

- Device LED lights up on startup accompanied by short “beep beep” from buzzer. Once LED turns off, it indicates device is ready for first calibration.

- phones/accessories are placed on at any time before step 4.

- Press “calibrate” button to read weight of devices loaded on

- LED double flashes and turns off once weight is read. Quiz Master is now activated

- Second button acts as a pause function to not alarm the device.  
Once calibrated this button requires a Long 5 second press to pause (for duration of the press it is accompanied by LED flashing and alarm double beeping for duration

- Once calibrated the calibrate button now also requires a Long 5 second press to recalibrate (for duration of the press it is accompanied by LED flashing and alarm double beeping for duration

- When device is out of weight, (For >2 seconds)  red LED flashes, alarm sounds and tick timer (TM1637) activates on side of Quiz Master. Once started this stays displayed and will pause when device is back in weight. This will continue to accrue until recalibrated. (Quiz Master can note down time if desired before recalibrating)

### External features

- 3d print a hard drive type tray that an external phone power bank can be placed into then slotted into the 3d printed shell (secured in undermount with couple screws) that can be charged seperately and provide power to peoples phones. Good win for people to like the product

- Space for >10 phones ( double depth phone slots) +deep tray for watches. iPads/laptops etc stay in bags